# 10.6 Backward Substitution

After completing the forward-substitution stage, the intermediate solution vector z becomes available. The final estimate of the transmitted signal vector is then obtained by solving the second triangular system

LHX=z

where:

- LH is the Hermitian transpose of the lower triangular matrix,

- X is the transmitted signal vector,

- z is the intermediate solution vector obtained from forward substitution.

Since LH is an upper triangular matrix, all elements below the main diagonal are equal to zero. This structure allows the unknown variables to be computed sequentially, but in the reverse order compared to forward substitution. [CIT-GOLUB]

For an N×N system, the matrix equation can be expressed as

l11*l21*⋯lN1*0l22*⋯lN2*⋮⋮⋱⋮00⋯lNN*x1x2⋮xN=z1z2⋮zN

where:

- lij* denotes the complex conjugate of the corresponding element in L,

- xi is an element of the transmitted signal vector,

- zi is an element of the intermediate solution vector.

The solution process begins with the last equation because it contains only a single unknown. The final element of the transmitted signal vector is therefore computed as

xN=zNlNN*

where:

- xN is the last element of the transmitted signal vector,

- zN is the last element of the intermediate solution vector,

- lNN* is the last diagonal element of the upper triangular matrix.

Once xN has been determined, the remaining variables can be computed successively by moving upward through the system. The general backward-substitution equation is

xi=1lii*zi−j=i+1Nlji*xj

where:

- xi is the current transmitted signal estimate,

- lii* is the diagonal element of the upper triangular matrix,

- lji* represents the upper triangular coefficients,

- zi is the corresponding intermediate solution element.

Similar to forward substitution, each computation depends on values obtained during previous iterations. However, the dependency direction is reversed. The solution process therefore proceeds from the last variable toward the first variable. [CIT-GOLUB]

Suggested Figure 10.6: Dependency graph of the backward-substitution process showing the sequential computation of xN,xN−1,…,x1.

Source: Author's work.

Caption Example:
Figure 10.6: Dependency graph of the backward-substitution process.

The dependency structure of backward substitution is summarized in Table 10.5.

| Variable | Depends On |
| --- | --- |
| xN | None |
| xN−1 | xN |
| xN−2 | xN−1,xN |
| ⋮ | ⋮ |
| x1 | x2,x3,…,xN |

Table 10.5: Data dependencies in the backward-substitution process.

From an algorithmic perspective, backward substitution completes the solution of the linear system and produces the final estimate of the transmitted symbols. Together, the forward- and backward-substitution stages replace the need for explicit matrix inversion while preserving the mathematical solution of the original equalization problem. [CIT-LUO]

The sequence formed by matched filtering, Gram matrix generation, Cholesky decomposition, forward substitution, and backward substitution constitutes the complete equalization algorithm adopted in this work. The integration of these stages into a unified processing flow is presented in the following section.