# 10.5 Forward Substitution

Following Cholesky decomposition, the original equalization problem is transformed into two triangular systems. The first of these systems is represented by

Lz=b

where:

- L is the lower triangular matrix obtained from Cholesky decomposition,

- z is the intermediate solution vector,

- b is the matched-filter output vector.

Since L is a lower triangular matrix, all elements located above the main diagonal are equal to zero. This property introduces a structured dependency pattern that enables the unknown variables to be computed sequentially from top to bottom. [CIT-GOLUB]

For an N×N system, the matrix equation can be written as

l110⋯0l21l22⋯0⋮⋮⋱⋮lN1lN2⋯lNNz1z2⋮zN=b1b2⋮bN

where:

- lij represents an element of the lower triangular matrix,

- zi is an element of the intermediate solution vector,

- bi is an element of the matched-filter output vector.

The first unknown can be computed directly because it depends only on the first equation:

z1=b1l11

where:

- z1 is the first intermediate solution,

- b1 is the first matched-filter output element,

- l11 is the first diagonal element of the lower triangular matrix.

Once z1 has been computed, it can be used to determine z2. The same procedure continues until all elements of the vector z have been obtained. The general forward-substitution equation is

zi=1liibi−j=1i−1lijzj

where:

- zi is the current intermediate solution element,

- lii is the diagonal element of the lower triangular matrix,

- lij represents the lower triangular coefficients,

- bi is the corresponding matched-filter output element.

The computation of each element depends on previously calculated values. Consequently, the solution process follows a strict sequential order. A variable cannot be computed until all variables on which it depends have already been determined. [CIT-GOLUB]

Suggested Figure 10.5: Dependency graph of the forward-substitution process showing the sequential computation of z1,z2,…,zN.

Source: Author's work.

Caption Example:
Figure 10.5: Dependency graph of the forward-substitution process.

The dependency structure of forward substitution is summarized in Table 10.4.

| Variable | Depends On |
| --- | --- |
| z1 | None |
| z2 | z1 |
| z3 | z1,z2 |
| ⋮ | ⋮ |
| zN | z1,z2,…,zN−1 |

Table 10.4: Data dependencies in the forward-substitution process.

From a hardware perspective, this regular dependency pattern offers several advantages. The computational sequence is deterministic, memory accesses follow a predictable order, and the required arithmetic operations consist primarily of multiply-accumulate and division operations. These characteristics simplify scheduling and facilitate efficient hardware implementation. [CIT-LUO]

In the equalizer adopted in this work, forward substitution constitutes the first solving stage following Cholesky decomposition. The generated intermediate vector z is subsequently used by the backward-substitution stage to recover the estimated transmitted symbols. The operation of this second stage is presented in the following section.