# 10.4 Matched Filtering

The transformation of the equalization problem into the linear system

HHHX=HHY

introduces two quantities that play a central role in the proposed equalizer: the Gram matrix HHH and the matched-filter output vector HHY. Before the system can be solved through Cholesky decomposition and triangular substitution, both quantities must first be computed. [CIT-LUO]

Matched filtering generates the vector

b=HHY

where:

- b is the matched-filter output vector,

- HH is the Hermitian transpose of the channel matrix,

- Y is the received signal vector.

This operation projects the received signal onto the channel basis and combines the information received across all antennas into a form suitable for symbol estimation. As a result, the matched-filter output serves as the right-hand side of the linear system that will later be solved by the equalizer. [CIT-LUO]

For an N×N MIMO system, the matched-filter operation can be expressed as

b=h1Hh2H⋮hNHY

where:

- hiH is the Hermitian transpose of the channel vector associated with the ith spatial stream,

- Y is the received signal vector,

- b is the matched-filter output vector.

Each element of the resulting vector is obtained through a complex inner product between a channel vector and the received signal vector. Consequently, the matched-filter stage consists primarily of complex multiplications followed by accumulation operations. [CIT-LUO]

Suggested Figure 10.4: Matched-filter operation showing the multiplication of the received vector by the Hermitian transpose of the channel matrix.

Source: Redrawn by the author based on [CIT-LUO].

Caption Example:
Figure 10.4: Matched-filter operation used to generate the vector b=HHY (redrawn by the author based on [Ref. XX]).

In addition to generating the vector b, the equalizer computes the Gram matrix

A=HHH

where:

- A is the Gram matrix,

- HH is the Hermitian transpose of the channel matrix,

- H is the channel matrix.

The Gram matrix captures the correlation between the spatial channels and forms the coefficient matrix of the linear system. Together, the matched-filter output vector and the Gram matrix provide all information required for the subsequent equalization stages. [CIT-LUO]

The complete preprocessing stage performed before solving the linear system is summarized in Table 10.1.

| Operation | Expression | Output |
| --- | --- | --- |
| Matched Filtering | HHY | b |
| Gram Matrix Generation | HHH | A |

Table 10.1: Preprocessing operations performed before equalization.

The outputs of the matched-filter stage are subsequently used to construct the linear system

AX=b

where the Gram matrix A forms the coefficient matrix and the matched-filter output b forms the right-hand-side vector. The solution of this system is obtained through Cholesky decomposition, which is presented in the following section.